Survey Operation Manual

Requirements of computer

CPU 2.50GHz，Internal memory 4G

The computer has at least one net port, a USB interface.

Installed Chinese 7/Windows8 Windows 32 bit, 64 bit operating system

Display resolution is recommended as 1440*900

Installation of survey

powerline

Internet cable

The signal line

Hardware

After link the powerline, the three lights will bright, the other end of singal line has two U style sighnal lines and black start button of trigger lines, has marked as “1” or “2”, as channel 1 and 2 respectively. The red insert spring is positive (+), the blue insert spring is negative. The signal line U is connected with the output signal of the chromatograph by the "connecting pile", and the identification channel number is noted and the polarity of the signal is maintained.

Installation of software

- Install the environment of the software, click to install the platform

- Click the Survy to install software,

3. Clickto open software.

PC

ClickWindows，Select "control panel" to enter "network and sharing center", click "local connection", enter property to use below IP address:

The last input range is 201-300.

Software operation

Open computer, and inset the key

Double clickto have below window.

User name is Admin, password is a12345

Instrument configuration.

Double click

Double clickto enter Instu1, through“»”，“ >”，“«”，“<”to add instruments.

Instuments

Doulbe clickto login in,choose project

Click right button to connect, click

If connect successully, it looks like or

进样序列

Click New sequence to enter to edit the sequence，clickto start acquire of data，then click to deal with data

SURVEY CDS

User Guide

Thank you for buying  SURVEY Chromatography Data System, in order to ensure your proper operation, please read this manual carefully, it has introduced the main performance and technical indicators, installation, unloading, debugging, and operation example, etc. If has any other questions, please contact with us for the timely answer.

Any product has shortcomings, our product is not exceptional. Please contact with us if you have any suggestion. Please contact us for improving the service.

1.0 Description

SURVEY software main window contains six functional modules, namely the instrument window, spectrum correction, spectra processing, user management, project management, equipment configuration, this six modules are used by the user in the daily sample acquisition and data processing, click the appropriate icon relevant login window will pop up, and each module has access right which can be edited by mananger. Also in the management of the icon bar there are four windows icons, namely language, help, IQ and version, click on the corresponding icon to view the module, no need to login, no need with authority.

The default user account is Admin, password is 123456. Click the "User Name" drop-down menu will pop up on the right, shows all users and passwords current software records, if you want to delete a user drop-down menu, you can select the user, click "clear", then the user will be removed from the drop-down menu.

1.1 Mainwindow

Left of the main window has seven system icons, the following were introduced corresponding function

| Icon | Name | Login | Function |
| --- | --- | --- | --- |
|  | User Management | √ | Perform user registration and permission settings, the role of the new and can be modified and reset user passwords |
|  | Project Management | √ | The project can be created, edited, operated and had audit trails |
|  | Instrument Configuration | √ | Management instrument configuration, add or remove instruments, view, modify equipment information |
|  | Journal | × | User can check the software operating record according to time, type, modules and operator, etc. |
|  | Language | × | Chinese, English and Japanese |
|  | Help | × | 3 kinds |
|  | IQ | × |  |
|  | Version | × | Display the version time |

1.1.1 User Management

After calling the User Management button and log in, there will be user management dialog box, which contains three parts: the user role, user information and personal information.

User Management

User information

User role

Basic user management procession:

Set up role

New, edit user name, password, choose role

If want to revise password, log in at the role and revise.

1.1.2 Program management

Projectis the data of methods, results and custom fields, view filters, defined by the user. The purpose of the project is to facilitate users to classify, manage, retrieve the appropriate data, before the start of each experiment you must first create or select an project.

4

3

2

1

1 Project structure

2 Status area

3 Display Area

4 Editor operating area

Project management includes two parts: my project and user projects.

Project management is for the new project, view, edit, manipulate.

"My Project" shows the project currently logged himself created, and "user program" is displayed for all users together to create the project.

Add two function, "Backup", "Restore." in the editing area.

- View

Choose one project, choose ”View” to display as below, includes the project information.

6

5

5 Operate record

6 Saved data path.

- Edit

- Operation

Choose Operate, can do revise to the project

- Audit

You can choose filter to find the data, and save in excel.

- New

Choose the parent group, and “New” to click the above.

- Backups of User Projects

- Restore of User Projects

1.1.3 Configue management

10

9

8

7

Four parts:

7 Instrument Display

8 nfo Display

9 System Configuration Display

10 Unit Display

Choosecan add a new instrument, choose  to save the change.  can define your own instrument. Image can be changed.

The instrument which connect to the CDS will displayed as

|  | Add one unit |
| --- | --- |
|  | Move one unit |
|  | Add all |
|  | Move all |

1.1.4 Diary

Log diary does not require user login, not require of authority. The main contents of the log records is the operation of software. According to time, type, modules and operator, etc., users can see the software operation records.

1.1.5 Language

1.1.6 Help

1.1.7 IQ

Installation Qualification: Confirm installation files, components, and installation process. Through installation, commissioning and on-site verification activities, offers a range of experimental data to prove the drug stability of test chamber installation and conditions, the which meet the design requirements, information and documentation meet the GMP requirements.

1.1.8 Version

Record the version and public date.

1.2 Instrument

Click the instrument image to log in, project can be choosed or not.

| Icon | Name | Effect |
| --- | --- | --- |
|  | New configuration | Click the icon, will pop up the "New configuration" dialog box, generally include control of the pump, detector and other equipment unit |
|  | New injection sequence | Edit injection sequence, if required it can be saved |
|  | Start Inject | Begin to collect the data |
|  | End inject | End the inject and save |
|  | End of Line | Stop and save, and does not affect other line / pin |
|  | End sequence | Save this acquisition, the row after the sequence row can not be collected |
|  | Abandon needle | If the line is set to gather several needle, discard the needle will not save the acquisitions, other will be usual. |
|  | Abandon the line | Abandon the current line do not save spectra |
|  | Give up sequence | Give up all sequence |
|  | Columns | Save the column information |
|  | Solvent alert | Set a low solvent, place the pump go empty |
|  | Zeroing | Baseline Zero |
|  | Wavelength scanning | Wavelength scan and energy scan |
|  | Baseline testing | Test baseline noise and drift |

- File Menu

There are seven choise on the bottom

IC, SS, MTD, CDF, SST, WS, files CAL.

IC :Configure

SS: sequence

MTD: processing

CDF: spectrum

SST: system suitability

WS: spectrum

CAL : calibration

Right click can export the files.

- Sequence

| Icon | Name | Effect |
| --- | --- | --- |
|  | Add | Click "Add" in the end of the sequence editing area, automatically add a line to default line status |
|  | Copay and Add | Select a line sequence, click the button, it will add to the end of the sequence |
|  | Delete | Delete |
|  | Insert | Insert |
|  | Up | Move up one line |
|  | Down | Move down one line |
|  | Start Inject | Start to collect data |
|  | End inject | End inject |
|  | End line | End line |
|  | End sequence | Save this acquisition, the sequence row after the row in the other samples are not collected |
|  | Abandon inject | If the line is set to gather several needles, discard the needle will not save the acquisitions, others as usual |
|  | Abandon line | Abandon the current line does not save spectra collected |
|  | Abondon sequence | Abandon the current line spectra collected and not collected after row |
|  | Print test | It can be used for test printing possible problems |

- Control window

Right click the corresponding district, the menu will display

Click the Connect, choose the port com, instrument can be connected.

Column information can be write on the below “current method” Refence button

- Pump current IC

Unit notice

1MPa=106Pa=10.1972kgf/cm2=10bar=145.038psi

1kgf/cm2=9.81 *10-2MPa=0.981bar=14.22psi

For quaternary pump, etc. The default state of the runtime system run channel A

Pump anti-control provide users with a solvent alert settings (you can set or not set), the main functions are to set the alarm solvent, solvent alert when value is less than the volume of solvent in the solvent bottle set, the pump will automatically stop (if in the process of collecting samples, sample collection ends automatically and save the current data collection), which can prevent pump cavitation phenomenon.

- Gradient setup steps:

The first step: In the upper right corner, pump selection area, edit solvents name, set the scale (mainly for quaternary pump, single pump proportion not set as 100%), and also includes a solvent alert settings. Pump front box is selected in the checked state.

Step Two: In the gradient value editing area (bottom left corner) to edit the gradient value, right-click the gradient value editing area can "add", "insert", "delete."

Third step: Then set the gradient map (bottom right)to set the end of the gradient pump state: the initial value, to keep the current. Click the "gradient map" to view the solvent gradient set scale drawing. After you finish editing, select "Save" or "Save" the ic Configuration

- Detector current IC

Program scan: the user can edit the length (ie, time spent in the wavelength), the wavelength value between 190 ~ 690nm. Scan results window displays the data acquisition, the scan results are not saved.

- Scanning and data acquisition window

Include two kinds of wave, pressure wave and data acquire wave.

Show grid: display grid preview

Show mark: Hook spectra show peaks split identity

- For dual-channel acquisition box, if the two channels are configured into the instrument, if you turn off the channel data acquisition window, click the button, you can make a baseline recovery.

- This function is for user-friendly comparison or reference to existing spectrum in the sampling process.

- Snap shot, Mainly online data quick save, but also provide users with additional data storage channel, users do not need to set the save path and file name to save, the system will default to the snapshot files saved in the root directory of the current project, and give a file system name

- The software can display "component results" for users. First you need to add MTD file to the sample, then in the sample collection procession, "component results" can be view. In addition, right-click the blank drop-down dialog box will pop up, the integrator can select items, click on the desired display of the integral term, can be automatically added to the dialog.

- Scan wave

Mainly used for absorbance detector scan and energy scan. Users need to edit start wavelength, end wavelength and selecting step. Software gives 0.25,0.5,1.0,2.0 four steps can be selected from the pull-down menu, additional user needs to set save path scanning the spectrum, if the user is not set, or forget to set save path, the software will be saved in the project under the root directory of the spectrum by default.

- Message window

Display status information configuration of all units of the instrument during operation. eg autosampler together will display the current status, vials number, number of injections, the remaining time; detector displays the switch status of the state, wavelength, frequency, and deuterium lamp; pump status, flow, maximum pressure is displayed, the minimum pressure and so on. If the instrument is under the gradient condition, you can click the right icon to pop the current sample gradient map.

- Message window

2.0 Data acquisition and processing flow

SURVEY software send the control and acquisition specification to instrument of HPLC by the group method, such as pump flow rate and wavelength of the detector. It also relies on a sample set method to specify the number of samples and the order to run, the function of each sample to be applied, the operating mode of the sample, balance of time before injection and so on.

2.1 Method Group

In SURVEY software, before collect data you must establish an appropriate method group.

Methods group is a collection of user-defined methods and channels, the method may include

- Instrument method used to collect data in order to collect the data (only operation)

- Apparatus and method for collecting and processing data processing (running and processing)

- Equipment and processing for collecting, processing and reporting data (running and reports)

- MTD and CAL file export method

For a detailed description of the different types of the following methods:

Instrument method used to specify the instrument control and data acquisition parameters. Such as traffic and wavelength. Equipment including pumps, solvent management system, detector, autosampler, sample management systems and gas chromatograph. The method can be set to the instrument to perform the following tasks:

      - Data collection

      -  Set the initial conditions to balance the system

      -  Balance Column

  -  Clean autosampler and detector flow cell line

Sample set method includes a set of functions and related information for the specified parameters from a set of samples in each vial (the standard sample or unknown sample) data collection.

Tip: If you run the same type of samples per day, you can create a sample group, simply modify the sample with a new name so that each sample run.

The method comprises processing a set of instructions that defines how the software SURVEY channel data from 2D or 3D channel processing. Optionally using a conventional data integration process. Processing method for performing the following tasks:

- Determine each peak by start and end the traditional method to integrate peak integration

- Custom define software calibration standards and unknown quantitative

- Chromatographic calibration curve each component and results

- Definition is correct component content and concentration of components

- Defined system suitability test calculation, and for each component define the permitted value range

- Definition the PDA processing parameters to generate peak purity and library match result

- Definition of chromatographic parameters matching process, the selected 2D chromatogram is designated as the reference chromatogram match, and enable the matching process chromatography

-  Calculation detector noise and drift parameters

Export method is the fule  format for exporting data. File formats include IC, SS, MTD, CDF or WS.

2.2 Data acquisition and processing

Processing and reporting of the results of the sample can be done in two ways

Method A, first establish a method group then sampling, processing and reporting the results automatically by the system and other operations

Method B, first establish the method group method, include the instrument and method,then the injection, and on the basis of the results obtained, establishment of processing results, reporting methods, calculate the results, print reports

2.2.1 Method A specific process

Logic of this method is relatively clear, very easy to understand for repeated analysis of a large number of laboratories, such as quality control (QC) labs.

Method B is not the key to establishing good all methods at once, but gradually establish a method group each method. For some research laboratories, since at the time of start of the analysis, the results of the sample to be analyzed to produce a do not know, can not be set in advance all all methods. In this case, you can use Method B process.

Note: The following we will be related to the introduction of B according to scheme. If you need to follow the procedures of Method A work, just reference to the following description, before running the samples all the way to the establishment of good.

2.3 Data Collection

Log in to the main window of the instrument can accomplish the following tasks:

- Control instruments, change the instrument parameters and save the changes to the instrument approach

- Real-time monitoring and view baseline data (or wavelength spectrum) of FIG.

- Load samples (single sample or sample set)

- Collected data from a single injection or sample set

- Print Preview spectrum

2.3.1 Running a single sample or sample group

Run single sample

It can be injection and data collection of a sample for one time. Typically performed when the need for rapid analysis of samples in this operation, wait the system reaches an equilibrium state, terminating observe baseline, edit the sample name, select the corresponding method group, to determine the running time. If the autosampler, you also need to select the injection volume and vial number, then click "Start" in his spare time, the sample run start

Run sample group sample

When several of samples collection data, you can use a set of conditions or instructions (called sample group). These conditions and instructions on how and when can collect data from each vial vial group. Samples may include a calibration standard and unknown samples.

You must define a sample set before collecting data. The method is to specify the number of samples and order, run-time functions to be performed on the sample, and the sample set method for each group of samples. For the entire sample group must specify run settings must be specified whether the sample group paused before running collection. If you want to use the sample group again, you can save it as a sample set method.

Samples run step

- User management, project management. Experiments confirm the user, whether logged in or New Admin user. If you need to run the sample project, first establish a project (if the project already exists, this step can be skipped); otherwise, you can select an empty project.

- Click on the pop-up instrument configuration dialogue of Instrument configuration module, configure the instrument unit needed.

- Click on the main window instrument instrument pictures, pop-up "load in" dialog box, edit user name and password, and click "OK", pop-up "select item" interface, you can select the project you have created, if the sample does not meet project requirements, you can select an empty project.

- Login to "data collection," connect the instrument unit, pop-up "Communication settings" dialog box, select the corresponding serial number of the instrument, click "Connect."

- New configuration IC, sequence SS, processing MTD (if there are conditions of the process set down, you can click the root directory, it will pop up the "Load", "Refresh" drop-down menu, click "Load", select the required profile or spectrum, loaded into the current project configuration file).

- Sample collection. Click the sampling window of hand loaded icons, pop-up "new sequence" edit box, edit autosampler parameter, file name, configuration

- File, select the bottle number, cleaning method ... When you are done, start the injection.

- Print reports. If it is desired to automatically print a report after the end of the sample run, you can check the "report" in the sequence of the edit box, the software will run on the samples after moving print the report.

Survey CDS

User Guide

Calibration

Calibration curve preparation

Calibration curve which is made under the calibration window, include external standard method or internal standard method. Concentration, system suitability can be calculated by the calibration file.

1. File

You can adjust the horizontal axis of the unit.

adjust spectrum level

That is a total of 20 correction system 20 calibration points, namely the components can have up to 20 calibration points, that a component can have up to 20 calibration points.

2.  components table

Right click the mouse at the table,

Click the column set,

3. Calibration Curve Made Procession

Calibration is done by analyzing good standard sample preparation specified calculate absolute component concentrations to determine response factors of the process.

Records: standard samples of the same concentration of parallel injections

Calibration point: from a different sample concentration calibration dots

Standard sample: also called standard sample or standard mixture, containing a known quantity of the sample amount of the compound

Standards can also be purchased from the national standard reagent suppliers

Calibration curve: a compound is a graphical representation of the amount and response data generated by one or more standard sample obtained.

3.1 ESTD

Open the first spectra, click add all the peaks , input the compound name,

change the Resp. Style to be Area or Height, and input the Amount correspondently.

Open another standard spectra, given the Cali. Dot. To 2. The first spectra can be closed or reserved. Click  , input the Amount correspondently.

Note: if the Resp. Style is 0, please check if the reserve time of spectra 2 is within the spectra 1.

Then open spectra 3, given the Cali Dot To 3. , click , input the Amount correpondently.

Then open spectra 4, given the Cali Dot To 4. , click , input the Amount correpondently

Click  in the left botton to see the calibration curve.

Menu above:

Response: Displayed the response value

Amount: Amount of compounds, which can be input in the table, the unit can be changed by click

Calcu Value: The amount value in the calculation mode

Depart: (calculation value-amount)/amount*100%

Recs: Number of Value which added in the same calibration point.

Use: √ means use this point,  will be displayed in the curve, if no choice,  will be displayed.

Resp. Style: Style can be choosed in this menu

Curve Style: Style can be choosed in this menu

Origin Style: Include three mode:

Ignore: ignore the point (0,0)

Join: join the point (0,0) as a point, which will be used when only one data point

Pass: pass the original point (0,0)

Remnant.: Resp. 1n(Resp.Value−Actual Value)2

3.2 ISTD

The basic step of ISTD is same with ESTD.

Notice: ISTD without equation.

Press to save the calibration file (*.cal).

Survey CDS

User Guide

Chrom

Spectra Processing

After complte the acquirsation o data, open the spectra at the below icron.

Introduction of Menu Bar

1. File

1.1 Files menu

File Browser   Click File , file name will appears at the left side of window; double click can open the spectrogram.

1.2

Three mode of spectrum can be opend, including * .cdf, Spectrum * .cdf , * .dat  and  * .ws .

- .cdf - CDS itself or in the same format spectrum

- .dat-N2000 format spectra

- .ws-spectrum file)

then select local maps saved, "local "means stored on a computer disk in this" project "is stored in the built project.

1.3 Export

You can export signal text (* .TXT), Fingerprinting (* .TXT), image (* .wmf). Exported files are in the export folder,

- Signal Text (* .TXT): data export to TXT text, including time and signal values

- Fingerprinting (* .TXT): this * .TXT file exported for the purpose of introducing traditional Chinese medicine chromatographic fingerprint evaluation system. Export by Fingerprinting (* .TXT) in TXT files can be opened directly in the chromatographic fingerprint similarity evaluation system in A.

If you want to open in the chromatographic fingerprint similarity evaluation system B, firstly exported to the control file (* .SCP) in the A spectrum, then you can open up the system B.

- Image (* .wmf): while export the spectrum, the spectrum can be exported to Word and Excel, open Word and paste. Each exported image file will be overwritten next time when you export the image file, so please save after exporting files at other files.

2. View

Top most: will always at the top page if click.

Version: can select the general and the TVOC version.

Option:

3. Method

Including Global Parameters, Manual Integral and Result sets.

4. Options

5. peak

Add positive peak

Add positive peak within the selected scope

Added Result

Add negative peak

Add negative peak within the selected scope

Delete Result

Delete peak

Detector delay

Manually select or input lag time value, spectra will be left or right offset by the input value time range, positive to left, negative to the right.

Global minimum peak width

Set the minimum peak width, smaller one will not be identified.

Global minimum peak height

Set the minimum peak width, smaller one will not be identified.

Negative peak detection

Shoulder cut area ratio

The default value of CDS is 3, two incompletely separate peak, if their area ratio

is more than 3, it was identified as shoulder peak, if less than 3 will be identified as overlapping peak.

Shoulder cut slope ratio

The default value is 2.5, two incompletely separate peak, if their slope is bigger than 2.5, it was identified as overlapping peak, if less than 3 will be identified as shoulder peak.

Seperation

According to the requirements two kindes can be choosed, automatic adjustment and unchanged baseline.

Baseline will be adjusted automatically after delete one peak.

Baseline will not be changed

Set smallest peak width

Set smallest peak height

Set smallest half peak width

Set smallest peak area

Valley point   Add or change the original valley position within a certain range

Make baseline across the valley point, adjust peak (group) to be single peak

Integration baseline, adjust the peak (group) as the overlapping peak

Adjust peak (group) the baseline level to be forward

Adjust peak (group) the baseline level to be backward

Before cut sets

Tail cut

Solvent peak

Group peak

Result

Noise evaluation, selection section in the spectrum of noise, the noise level will be displayed below.

- Drift assessment, selection drift period in the spectrum, drift values displayed below.

6. Effectiveness

Contains some form peak performance parameters, which are calculated here (EP) 50% peak width in accordance with the European Pharmacopoeia. Non-peak retention time and column length can be adjusted according to the actual situation, the value will change in response, such as column efficiency / L items, and the separation degree.

7. summary

When you open two or more than two spectra, input the same component name for the same component, or load the calculation file in the result set, it will automatically calculate the average of the same components, include the standard deviation and% RSD and so on.

8. SST

SST: To verify the chromatography system based on the chromatographic evaluation. Load the files at the opend spectra, components will be refreshed in the calibration file.

|  | Component selection did not enter any value, to be edited. |
| --- | --- |
|  | All parameters are in line, they were within range. |
|  | Parameter exceeds a given limit, do not match. |

9. Spectrum

Sensitivity values can changed by .